|  | Simulation of Swiss Cube Date: Thursday, March 12, 2020 Designer: Arjun Singh Study name: SimulationXpress Study Analysis type: Static. Table of Contents Description 1 Assumptions 2 Model Information 2 Material Properties 3 Loads and Fixtures 3 Mesh information 4 Study Results 6 Conclusion 8. |
| --- | --- |
| # Description No Data |  |

# Assumptions

# Model Information Model name: Swiss Cube Current Configuration: Default. Solid Bodies. Document Name and Reference; Treated As; Volumetric Properties; Document Path/Date Modified. LPattern4; Solid Body; Mass:0.119651 kg Volume:4.43152e-05 m^3 Density:2,700 kg/m^3 Weight:1.17258 N Density = 0.10 pounds per cubic inch Mass = 0.26 pounds Volume = 2.70 cubic inches (34.328) Surface area = 81.64 square inches (63.6750); C:\Users\Acer\Documents\Swiss Cube.SLDPRT Mar 12 13:44:58 2020.

# Material Properties Model Reference; Properties; Components. Name:; 6061 Alloy. Model type:; Linear Elastic Isotropic. Default failure criterion:; Unknown. Yield strength:; 5.51485e+07 N/m^2. Tensile strength:; 1.24084e+08 N/m^2.; SolidBody 1(LPattern4)(Swiss Cube).

# Loads and Fixtures Fixture name; Fixture Image; Fixture Details. Fixed-1; Entities:; 1 face(s). Type:; Fixed Geometry.. Load name; Load Image; Load Details. Force-1; Entities:; 1 face(s). Type:; Apply normal force. Value:; 250 lbf..

# Mesh information Mesh type; Solid Mesh. Mesher Used:; Standard mesh. Automatic Transition:; Off. Include Mesh Auto Loops:; Off. Jacobian points; 4 Points. Element Size; 0.180719 in. Tolerance; 0.00903595 in. Mesh Quality Plot; High. ## Mesh information - Details Total Nodes; 42650. Total Elements; 17775. Maximum Aspect Ratio; 30.835. % of elements with Aspect Ratio < 3; 51.8. % of elements with Aspect Ratio > 10; 4.27. % of distorted elements(Jacobian); 0. Time to complete mesh(hh;mm;ss):; 00:00:30. Computer name:.

# Study Results Name; Type; Min; Max. Stress; VON: von Mises Stress; 5.505e+03 N/m^2 Node: 31260; 1.474e+08 N/m^2 Node: 11215. Swiss Cube-SimulationXpress Study-Stress-Stress. Name; Type; Min; Max. Displacement; URES: Resultant Displacement; 0.000e+00 mm Node: 3503; 3.039e-02 mm Node: 22608. Swiss Cube-SimulationXpress Study-Displacement-Displacement. Name; Type. Deformation; Deformed shape. Swiss Cube-SimulationXpress Study-Displacement-Deformation. Name; Type; Min; Max. Factor of Safety; Max von Mises Stress; 3.742e-01 Node: 11215; 1.002e+04 Node: 31260. Swiss Cube-SimulationXpress Study-Factor of Safety-Factor of Safety.

# Conclusion